# Getting Started

### Telcom FAX Server 1.5

LTCTMTelcom FAXTM

for Windows NTTM

Advanced 32-bit FAX Software

Copyright © LTC 1993-1994.  All rights reserved.

LTC and Telcom FAX are trademarks of LaRonde Technical Consulting.
Windows NT is a trademark of Microsoft Corporation.

# Contents

µChapter 1	Introduction	1

Chapter 2	Installing Telcom FAX	2

System Requirements	2

Running Setup	2

Configuring Telcom Server	3

Chapter 3	Using Telcom FAX Administrator	6

Chapter 4	Sending a FAX Message	7

Chapter 5	The Print-to-FAX printer driver	9

Installing the Print-to-FAX printer driver	9

"Printing" to a FAX machine	11

Appendix A	Technical Information	12

Appendix B	Compatibility	13

Appendix C	Technical Support	14

Appendix D	Credits	15

# Chapter 1	
Introduction

Welcome to Telcom FAX Server for the Windows NT and Windows NT Advanced Server operating systems.  Telcom FAX Server enables your entire network, including Windows NT and Windows for Workgroups clients, to send and receive FAX messages using a shared network FAX server.

Telcom FAX Server is based on the client-server model.  Also, it's written entirely in C++, using object oriented programming techniques throughout.  Because of it's object oriented design, LTC plans to able to continue to add features to Telcom FAX Server quickly and efficiently.

Please read this document before installing Telcom FAX Server. It contains step-by-step information about installing Telcom FAX Server.

# Chapter 2	
Installing Telcom FAX

## System Requirements

The requirements for Telcom FAX are:

·	Windows NT or Windows NT Advanced Server operating system version 3.1

·	Class 1, Class 2, or Class 2.0 compatible FAX-modem

·	Modem cable that supports all modem control signals

## Running Setup

Before you can start using Telcom FAX Server, you need to run the Setup program provided in the SERVER directory on the distribution disk. This program copies the program files to your hard disk, installs the Telcom Server service, and makes the necessary entries in your registry.

Before you run the Telcom FAX Server Setup program, log in with a user account that belongs to the Administrator group, and verify that Mail is working correctly for that user.  The Setup program accesses Mail configuration information in the registry.  If Mail is not set up correctly, then the Setup program will not be able to fully install the software.

### To run the Telcom FAX Server Setup program

1.	Make sure you are logged into Windows NT with a user account that belongs to the Administrator group for your computer.

2.	Make sure that Mail is working properly.

3.	Insert the Telcom FAX Server distribution disk in drive A (or B), or if you are installing from a hard disk, be sure that all of the distribution files are all in one directory on your hard disk.

3.	Open the Windows NT File Manager.

4.	Select the drive and directory where the distribution files are located (for example, A:\SETUP, B:\SETUP, etc.)

5.	Run the Setup program by double-clicking on SETUP.EXE. Or, if you prefer, use any other method to run SETUP.EXE.  Note: be sure your current directory isn't the Windows NT "SYSTEM32" directory, or Setup may overwrite the Windows NT "setup.inf" file.  If this happens, you can restore "setup.inf" from the original Windows NT distribution disk.

6.	Follow the instructions on the screen.

7.	When Setup is complete, proceed to "Configuring Telcom Server".

## Configuring Telcom Server

Note: if you haven't run Setup yet, do that first (see Running Setup).

The following is just a step-by-step guide for configuring Telcom Server for the first time.  For more information on configuring Telcom Server, please see the "Settings dialog" topic in the Telcom Administrator Help document.

### Creating a Telcom mailbox

Telcom Server uses an outgoing FAX message mailbox.  Mail users send outgoing FAX messages to this mailbox.  Telcom Server monitors this mailbox, and transmits the outgoing FAX messages to the destination FAX number.

Telcom Server works best if the outgoing FAX message mailbox is set to check for new messages every one (1) minute.  Also, you should configure it not to save copies of outgoing messages.

### To set up an outgoing FAX message mailbox

1.	Sign on to Mail with the Post Office Administrator account.

2.	From the Mail menu, choose Postoffice Manager.

3.	Press the Add User button.

4.	Fill in the Name, Mailbox, and Password boxes with telcom.

5.	Press the OK button.

6.	Press the Close button.

7.	From the File menu, choose Exit and Sign Out.

### To set the outgoing message mailbox options

1.	Sign on to Mail as telcom, password telcom.

2.	From the Mail menu, choose Options.

3.	Un-check the box labeled "Save copy of outgoing messages in Sent Mail folder".

4.	Fill in the box labeled "Check for new mail every _____ minutes" with 1.

5.	Press the OK button.

6.	From the File menu, choose Exit and Sign Out.

### To set up the Telcom Server MAPI Gateway properties

Note:  if you chose the name telcom, password telcom, for your outgoing FAX mailbox, you can skip over this procedure.

1.	Open the Telcom Administrator (also called Telcom Admin, or just Admin). You can use the icon that Setup installed in your program manager to do this.

2.	From the Server menu, choose Gateways.

3.	Fill in the Mailbox and Password blanks with the name of your outgoing FAX message mailbox.  The boxes are both set to telcom by default.  This is the suggested configuration, but you can change it if you like.  Be sure that the mailbox exists, and that you have typed in the correct password for the mailbox.

4.	Fill in the blank labeled "Check every ___ seconds" with the number of seconds you want the gateway to pause between operations.  The recommended setting is ten (10) seconds.

5.	Press the OK button to keep your changes.

### To set up the Telcom Server devices

1.	Select FAX-modem from the devices window.

2.	From the Device menu, choose Properties, or double-click on the device.  This brings up the Device Properties dialog.

3.	Select a driver that will work with your modem from the "Driver" list.  You might try sending FAX messages with different drivers selected to determine which one works best with your modem.

4.	Fill in the Description blank with a description for this device (optional).

5.	Fill in the Local ID blank with your station identifier (for example, 315-471-7332).  You can type in up to twenty characters.

6.	Check the box labeled "Call out with this device" if you want to send FAX messages with this device.

7.	Fill in the Dial prefix blank with any special pre-dial requirements you have (for example, use *70, to disable call waiting for one call, or 9, to get to an "outside" line if you are on a PBX). Telcom Server assembles the command string AT D<P,D> <pre-dial><fax number> and sends it to the modem to dial the phone. See your modem documentation for more information about dialing and the AT D command.

8.	Check the box labeled "Receive calls with this device" if you want to receive FAX messages with this device.

9.	Fill in the blank labeled "Route messages to ____" with the name of the mailbox that you want to receive inbound FAX messages.

10.	Check the box labeled "Keep a journal for this device" if you want Telcom Server to track this device's operations in a journal file.  Journaling is used for troubleshooting, and should generally be disabled.

11.	Fill in the box labeled "Journal file" with the name for the journal file.  If you don't specify a path, the file will be created in the directory where you installed Telcom Server.

12.	Push the Settings button.

13.	Fill in the Port blank with the name of your FAX-modem port (for example, COM2).  You can select COM1 through COM4 from the list, or you can type in a specific port name.

14.	Fill in the Options blank with any special modem commands you need to send to your modem.  See Appendix A for settings specific to your modem.  Leave off the AT prefix.  Telcom Server assembles the command string AT <init> and sends it to the modem as one of the last steps in the initialization sequence.
Don't put Z or &F, or any other "reset" type commands in this blank.  Telcom Server issues an AT&F as part of it's own initialization.  Telcom Server expects the modem to have hardware (RTS/CTS) enabled.  If your modem doesn't enable hardware (RTS/CTS) flow control when the AT&F command is issued, you need to enable it in Options.  For examples of enabling hardware flow control, see "Issues" at the end of this manual.  Please refer to your modem documentation for a list of  AT commands for your modem.

15.	Fill in the Max bps blank with the maximum speed at which you want the device to try and establish carrier.

16.	Adjust the Dialing, Speaker, and Volume settings as desired.

17.	Press the OK button to keep your changes.

18.	Press the OK button to end the Device Settings dialog.

You are now ready to start using Telcom FAX Server.  You may want to install the Telcom FAX Print-to-FAX printer driver on your licensed Windows NT client machines.  This will enable them to include output from almost any application in FAX messages.

# Chapter 3	
Using Telcom FAX Administrator

### Starting & Stopping the Server

Choose start from the "Server" menu in Telcom Administrator to start Telcom Server.  If the server starts properly, the "Start" choice in the server menu will be checked, and the start button on the toolbar will appear pressed.

To stop the Server, choose "Stop" from the "Server" menu.  The "Stop" choice in the server menu will be checked, and the stop button on the toolbar will appear pressed.

If the Server doesn't start properly, check in the System event log using Event Viewer for Telcom error messages.  Also, the files TSRV.JNL, QUEUE.JNL, and DEVICE.JNL in the \WINDOWS\SYSTEM32 directory contain a complete record of the most recent server session.

Please refer to Telcom Administrator's on-line help documentation for a more complete discussion on using Telcom Administrator.

# Chapter 4	
Sending a FAX Message

### To send a FAX message using Microsoft Mail

1.	Start Microsoft Mail.

2.	From the Mail menu, choose Compose Note.

3.	Fill in the To blank with the name of the outgoing FAX message mailbox (for example, telcom).

4.	Leave the CC blank empty.

5.	Fill in the Subject blank with the appropriate subject (whatever you like).

6.	In the message body, type in /to <recipient>@<FAX#> and then press ENTER, where <recipient> is the name of the FAX recipient, and <FAX#> is the phone number of the destination FAX machine.  You need to include a space after the /to, and after all other "slash" switches.

7.	Type in the text you want to appear on the cover page.  Type this text in after the addressing information on the first line of the body.  This text will appear below the header information, and is limited in length to what will fit on the cover page.

8.	Attach one or more DCX files if you like.

9.	Press the Send button to send the message.

### To attach one or more DCX files to a FAX message

1.	Prepare the mail message as in "To send a FAX message" above.

2.	Before pressing the Send button, press the Attach button.

3.	Select a DCX image you want to send.

4.	Press the Attach button.

5.	Repeat steps 2 & 3 to attach additional DCX files (up to thirty).

6.	Press the Close button.

7.	Press the Send button to send the message.

### To use the Microsoft At Work FAX printer driver

1.	Open a document in the application of your choice.

2.	Select the "Microsoft At Work FAX on FAX:" printer driver.

3.	Print the document.

4.	Complete steps 3 through 7 in "To send a FAX message" above.

### Additional FAX addressing switches

/voice <#>	<#> is sender's voice # for cover page

/fax <#>	<#> is sender's FAX # for cover page

/try <n>	Attempt transmission <n> times

/wait <n>	Wait <n> minutes between transmission attempts

/at <hh:mm>	Schedule transmission at <hh:mm>

/nocover	Do not send a cover page

/confirm	Return a confirmation message

/discard	Do not return the message if it cannot be sent

/returnto <mb>	Return messages to mailbox <mb>

# Chapter 5	
The Print-to-FAX printer driver

## Overview and Installation

Use the Telcom FAX Server Print-to-FAX printer driver to send practically anything you can print to a FAX machine. The driver lets you "print" to any FAX machine as if were a printer connected directly to you own computer or network.

You should only install and use the Print-to-FAX printer driver on Windows NT client workstations that you have purchased licenses for.  You will not be able to use it at the same time on the same machine as Telcom Server, due to the nature of services and MAPI.

You can install the driver on the server machine (where Telcom Server is installed), but you can't use it while Telcom Server is running.  This is due to a limitation in MAPI.  We may provide a solution to this problem in the near future.  To use the Print-to-FAX driver on the server machine, first stop Telcom Server, then use the driver, then start Telcom Server again.

### To install the Print-to-FAX printer driver

1.	Make sure you are logged into Windows NT with an administrator account.

2.	Insert the Telcom FAX distribution disk in drive A (or B), or if you are installing from a hard disk, be sure that all of the distribution files are in the same directory on your hard disk.

3.	Open the Windows NT File Manager.

4.	Select the drive and directory where the distribution files are located (for example, A:\PTF, B:\PTF, etc.)

5.	Run the Setup program by double-clicking on SETUP.EXE. Or, if you prefer, use any other method to run SETUP.EXE.  Note: be sure your current directory isn't the Windows NT SYSTEM32 directory, or Setup may overwrite the Windows NT setup.inf and printer.inf files.  If this happens, you can restore setup.inf and printer.inf from the original Windows NT distribution disk.

6.	Follow the instructions on the screen.

7.	When Setup is complete, proceed to "To add the Telcom Print Monitor".

### To add the Telcom Print-to-FAX Monitor

1.	In the Windows NT File Manager, select the drive and directory where you installed the Print-to-FAX driver files (for example, C:\TELCOM\PTF).

2.	Run the Print Monitor Manager program by double-clicking on PRINTMON.EXE.

3.	Press the Add button.

4.	Type Telcom Monitor in the Name box.

5.	Press the Browse button.

6.	Double-click on PTFMON.DLL.

7.	Press the OK button.

8.	Press the Close button to close the Print Monitor Manager.

### To create the Print-to-FAX printer

1.	Open the Windows NT Print Manager.

2.	From the Printer menu, choose Create Printer.

3.	Type in a name for the printer (for example, FAX, or Telcom FAX Server).

4.	From the Driver list, select "Other..." (it's all the way at the bottom).

5.	Specify the path where the Telcom FAX Print-to-FAX files were installed (for example, C:\TELCOM\PTF).

6.	Press the OK button to accept the path.

7.	Press the OK button to select Telcom FAX Print-to-FAX.

8.	Specify the path of your Windows NT distribution disk and press the Continue button.   Some files will be copied to your hard disk, and Telcom FAX Print-to-fax will be added to the printer driver list.

9.	Type in a description for the printer (for example, the FAX-modem on my desk).

10.	Select PTF from the "Print to" list (it may already be selected).  If PTF is not in the list, you need to install the Telcom Print-to-FAX Monitor.  See "To add the Telcom Print-to-FAX Monitor" for more information.

11.	Press the OK button to finish with the Create Printer dialog.

12.	Press the OK button to select Letter as the paper size.

## "Printing" to a FAX machine

"Printing" a document on a remote FAX machine isn't very different at all from printing a document on your local printer. The most significant difference is that you'll need to specify the telephone number of the FAX machine that you want to "print" to.

### To print to a FAX machine

1.	Open a document in the application of your choice.

2.	From the File menu, choose Print Setup (or the equivalent choice).  If there is no Print Setup choice in the File menu, then you may find it under the Print choice.

3.	Select the Telcom FAX Print-to-FAX printer you installed.

4.	Press the OK button to return to the document.

5.	From the File menu, choose Print.  A conversion dialog box will has appear.  This will eventually close and a Mail signon box will appear.

7.	Sign on to Mail with your normal ID and password.  Don't use the telcom (outgoing FAX message) mail account to send FAX messages.

8.	When the Send FAX Message dialog appears, fill in the blanks to compose the FAX message.

9.	Press the Send button to send the FAX message.  At this point, your FAX message will be mailed to Telcom Server.  From there, Telcom Server will deliver it to the destination FAX machine.

# Appendix A	
Technical Information

Here are some know issues, work-arounds, and tips.  Please note that any references to planned features in "future" versions of Telcom FAX Server are speculative and subject to change without notice.

LTC is interested in your feedback.  At your convenience, please send us CIS mail @ [71621,637] (internet 71621.637@compuserve.com) and let us know about any problems or suggestions you may have.

### Tested modems & settings

Modem	Class	Setting	Value	Reason

Banksia MyModem SOHO Series	1,2	None

BOCA M1440I	1,2	Options	&R0	RTS/CTS f/c

Cardinal 14400	1,2	None

Hayes Accura 144 + FAX	1	None

Hayes Accura 288 V.FC + FAX	1	None

Hayes Optima 28.8	1	None

Intel SatisFAXtion Modem/400e	1	None

PPI PM14400FXSA	1,2	None

USR HST DS w/FAX, ASL Terbo	1	Options	&H1	CTS f/c

USR Sportster 14,400	1	Options	&H1	CTS f/c

Switches	1U 2U 3D 5D 6U 8D	power on defaults

Supra FAXmodems	1,2	Options	&K3	CTS f/c

Telebit T3000/Wordblazer	2	Options	S58=2	full duplex RTS/CTS f/c

Megahertz	1	Options	\Q3\J0	CTS f/c, fixed serial rate

Motorola UDS FasTalk FAX 32bx	1	Options	\Q3	CTS f/c

ZyXEL U-1496	2	None

### Sending a "Fine" fax to a "Standard only" FAX machine

If you send a "Fine" fax to a FAX machine that only supports "Standard" resolution, the FAX may "stretch" vertically to approximately double its original size.  Currently, there is no work-around for this, although in our experience this is a rare situation.  LTC intends to include better handling for this case in a future release of Telcom FAX Server.

### Printing a FAX image (DCX file) from the FAX Viewer

The Viewer does not "fit" the fax to the printed page.  The very bottom portion of a FAX message may be missing on the printout.  LTC plans to fix this in a future release.

# Appendix B

# Questions & Answers

Q.  When I try to run Mail when Telcom Server is running, I get the message "Out of memory".

A.  Telcom Server and Mail both use MAPI.  Due to the nature of MAPI and services, you can't run Telcom Server and Mail at the same time.  The "Out of memory" message put up by Mail is misleading - it has nothing to do with memory.

If you need to use Mail on the same machine as Telcom Server, you need to stop Telcom Server, use Mail, then start Telcom Server again when you are through.  This won't hurt anything as long as FAX communication isn't in progress.  Ideally, Telcom Server should be on a machine that is not used for Mail.

Q.  Will Telcom work on Daytona (Windows NT 3.5)?

A.  Yes, but error messages from MAPI during startup won't be displayed - rather, they are "displayed" by Daytona on a "hidden" desktop that is not accessible.  If there is an error, it is difficult to tell what it is.  As long as Telcom Server is properly set up, it should start and run fine.

Q.  How can I tell better what's going on inside of Telcom Server.

A.  Look at the .JNL files in the installation directory with a text editor (like notepad).  They are transcripts of the various functions within Telcom Server.  The device journal can be turned on and off and renamed from within the Device Properties dialog box in Telcom Admin.

Q.  What's the difference between Class 1, 2, and 2.0?

A.  The FAX class refers to the command language between the modem and the FAX software.  Class 1 was the first standard with about ten commands.  Class 2 is based on a draft of a standard, with many more commands, but it varies from between manufacturers.  Class 2.0 is the "real" Class 2 standard in its final form.  It doesn't really matter which class you use.  Pick the one that works best with your modem.

Don't confuse "class" with "group".  Group refers to the telecommunication standard used between FAX devices.  Most FAX devices are Group 3.

Q.  How can I tell what class my modem is?

A.  In Terminal, type AT+FCLASS=?.  The modem will respond with a list of classes your modem supports.  Class 0 is data mode.

# Appendix C	
Technical Support

LTC will provide registered users ninety (90) days of telephone support free of charge, measured from the day of your first support call.

Please send the following information, or have it available when you call:

·	your FAX-modem brand

·	your FAX-modem model

·	your FAX-modem firmware version number

·	the result of the "AT+FCLASS=?" command

·	the communications port your FAX-modem is installed on

·	a detailed description of the problem(s) you are experiencing

### Compuserve Mail - preferred method

Send a mail message to LTC [71621,637]

Internet users, send a mail message to
	71621.637@compuserve.com

### FAX

Send FAX messages to (315) 478-0316

### Telephone

Call LTC @ 315-471-7332, M-F, 8-5 EST

# Appendix C	
Credits

### Thanks...

To my wife, Anita, who has supported me all the way; to Steven Diederich for spending so much time during the development of Telcom FAX; to the key beta testers Randy Braze, Bob Chronister, John Coupens, Jeff Mason, Tony Neczet, William Park, Stephen Purpura, Mike Rosado; to Mark Worsnop with his nifty scope; to Robert Wittnebert, who convinced me to integrate with mail; to Jeff Rosenthal and his help with the Alpha port; and to everyone else who spent their time, efforts, and computer resources to make Telcom FAX a reality.